UHF API Instructions

- Copy .jar and .so to your project：

- Eclipse：

- Android studio:

- Call UhfManger.class methods：

| 1.4 | UHF-ISO1800-6C | UhfManager |  |
| --- | --- | --- | --- |
|  | 1.4.1 | getInstance() | 6 |
|  | 1.4.2 | getFirmware() | 6 |
|  | 1.4.3 | setOutputPower() | 6 |
|  | 1.4.4 | inventoryRealTime() | 6 |
|  | 1.4.5 | selectEPC() | 7 |
|  | 1.4.6 | readFrom6C() | 7 |
|  | 1.4.7 | writeTo6C() | 7 |
|  | 1.4.8 | setWorkArea() | 7 |
|  | 1.4.9 | getFrequency() | 7 |
|  | 1.4.10 | setFrequency() | 7 |

1.4  UHF-ISO1800-6C

1.4.1 getInstance()

| Function | UhfManager getInstance() |
| --- | --- |
| Description | Get Uhf manager instance and open device |
| a |  |
| Parameter | void |
| Return | Uhf manager instance |

1.4.2 getFirmware()

| Function | byte[] getFirmware() |
| --- | --- |
| Description | Get firmvare version |
| a |  |
| Parameter | void |
| Return | Version information |

1.4.3 setOutputPower()

| Function | boolean setOutputPower(int value) |
| --- | --- |
| Description | Set antenna power gain |
| a |  |
| Parameter | Power value |
| Return | True: set success False: set failure |

Note: Scope of value.

1.4.4 inventoryRealTime()

| Function | List<byte[]> inventoryRealTime() |
| --- | --- |
| Description | Real-time get uhf epc list |
| a |  |
| Parameter | void |
| Return | Epc list |

1.4.5 selectEPC()

| Function | void selectEPC(byte[] epc) |
| --- | --- |
| Description | Select epc |
| a |  |
| Parameter | epc |
| Return | void |

1.4.6 readFrom6C()

| Function | byte[] readFrom6C(int memBank, int startAddr, int length, byte[] accessPassword) |
| --- | --- |
| Description | Read tag data |
| a |  |
| Parameter | Membank , start address , read length , password |
| Return | Tag data |

1.4.7 writeTo6C()

| Function | boolean writeTo6C(byte[] password, int memBank, int startAddr, int dataLen, byte[] data) |
| --- | --- |
| Description | Write data into iso-1800-6c tag |
| a |  |
| Parameter | password , Membank , start address , data length , data |
| Return | True: write success False: write failure |

1.4.8 setWorkArea()

| Function | int setWorkArea(int area) |
| --- | --- |
| Description | Set device work area |
| a |  |
| Parameter | Work area |
| Return | 0:set success Other: set failure |

Note: American standard by default. Generally do not need to be modified.

1.4.8 getFrequency()

| Function | int getFrequency() |
| --- | --- |
| Description | Get module work frequency |
| a |  |
| Parameter | void |
| Return | Frequency |

1.4.8 setFrequency()

| Function | int setFrequency(int startFrequency, int freqSpace, int freqQuality) |
| --- | --- |
| Description | Set frequency |
| a |  |
| Parameter | Start Frequency, frequency Space, frequency Quality |
| Return | 0:set success Other: set failure |

Note: American standard by default. Generally do not need to be modified.